Module: R5: RV-fpga

Section: Installations Task: Tools

Task 1.2

RVfpga-ViDBo

- Testing:

  - RVfpga-ViDBo

    - For using this simulator in Ubuntu it is required to first install websockets library, which can be easily achieved by executing the following command in a terminal:

sudo apt-get install -y libwebsockets-dev

    - Opened the specified folder of example program in VS Code:

    - PlatformIO will now open this program, LedsSwitches_C-Lang, that reads the switch values on the Nexys A7 board and writes their value onto the LEDs on the board. We can view the LedsSwitches_C-Lang program by expanding the src folder and double-clicking on LedsSwitches_C-Lang.c.

    - Open file platformio.ini. Then, established the path to the provided RVfpga-ViDBo simulator binary by editing the following line (replaced [Path-To-RVfpga] with the appropriate path in my system and chose the binary that corresponds to my Ubuntu version):

board_debug.verilator.binary = /home/xe-user106/RVfpga/Simulators/verilatorSIM_ViDBo/OriginalBinaries/RVfpga-ViDBo_Ubuntu22

    - Executed the RVfpga-ViDBo simulator from PlatformIO:

      - Click on the PlatformIO button on the left side.

      - Expand Project Tasks > env:swervolf_nexys > Platform and clicked on RVfpga-ViDBo/Pipeline.

This first compiles the program and then launches the Verilator simulation of the RVfpga SoC running this program.

    - Once the RVfpga-ViDBo is executing, I launched the python server simulating the Nexys A7 board.

    - Open a terminal. Just go into the /RVfpga/Simulators/verilatorSIM_ViDBo directory.

Execute the python server by running the following command:

python3 -m http.server --directory NexysA7board/.

    - Open a browser and connect to http://localhost:8000/nexys-a7.html.

Click on Connect to the board and test the program.